An Introduction to

3D Design and Engineering

With

Pro/DESKTOP™

Standards:

A.4.6, A.8.2, 8.3, B.4.6, 4.7, B.8.3, 8.7, C.4.3 , 4.4 , 4.5, 4.6 , 4.7, C. 8.5 , 8.6

A4.8

ProDesktop is a program used in industry to design complex shapes and parts. It allows individual pieces to be designed in 3D and then assembled to make a virtual model of a product.  Let’s Begin with a simple shape:

1.  Double click on the ProDesktop icon

2.  Select Close on Tip of the Day

3.  Click on the “New Design” icon

This will create a new design area.

**(If your window is small, click on the “maximize” button)

Lineee

Circle

Rectangle

Square

Arc

Spline

Cutting tool (Delete button)

------------------------------------------------------------------------------------------------------------

Before you begin:  See Appendix A (located in the back of this packet)

Drawing a simple base shape.

1.  Select the rectangle tool from the toolbar  (See Below)

2. Click and hold down the left hand mouse button at the corner of where the two arrows meet (see below)

3.  Drag the shape to any desired size and then release mouse button.

Corner

Rectangle Tool Bar

Dimensioning

You can change the size of a shape.  The following will show you how:

1.  Select the Sketch Dimension tool  (located at the top)

2.  Move the mouse over the top line of the shape until the line changes color

3.  Click and hold down the mouse button and Drag 1 dimension out to left side AND 1 dimension out of the top (see below).

Sketch Dimension Tool

4.  Make sure the “Constraints” button  is selected

5.  Double click of one of the numbers (a dialog box appears)

6.  Change the value to 5 inches

7.  Click OK

Open your “Test Packet” and answer all questions in Part A

MAKING A SHAPE 3 DIMENSIONAL (3D)

1.  Click on the extrude profile button  (located at the bottom of your screen).

2.  In the dialogue box that appears ensure that:

“Add material” is checked

“Above workplane” is checked

3.  Type in 5 for the height of extrusion

4.  Click  OK.

To change the extrusion:

If you did not want the height of the box to be 5 inches but instead wanted it to be 4 inches, you could easily change the height.  The following will show you how:

1.   Locate the browser dropdown window located in the upper left corner of the screen and click on the down arrow (see below)

2.  Select the “Features” option

3.  Double click on the “extrusion 1” icon  (A dialog box appears)

4.  In the dialog box that appears, change the extrusion from 5 to 4.

5.  Click “OK”

6.  VERY IMPORTANT:  You must now click on the “Update” button Located at the top of the screen (This updates the changes that you just made)

Open your “Test Packet” and answer all questions in Part B

MAKING CURVED EDGES

1.  Click on the edge tool

2.  Click on one of the edges (until it turns red)

3.  Click on the round edge tool.

3.  Type in a 1 for the radius.

4.  Click on OK

BUILDING ON TOP OF A 3D SHAPE

1.  Click on the face button

2.  Move your mouse over the top face until the outline of the face changes color then Click on the top face (the face will turn red when selected).

3.  Click on the Workplane menu (see below).

4.  Click on New Sketch option.

5.  In the dialogue box, click on OK

6.  Click on the circle tool button.

7.  Click and drag to a 2” diameter

8.  Release mouse button.

The circle shape can now be extruded (made 3D) to any desired height as described earlier. Using what you have learned, extrude the circle to a height of 3 inches.

Open your “Test Packet” and answer all questions in Part C

Adding a Hole

Holes can be added to any part.  Let’s start by creating a new shape:

1.  Start a new design  (Do NOT save the previous design)

2.  Create a rectangle that is 5” long by 3” wide.  (see below)

IMPORTANT: Make sure you drag out the dimensions

3. Extrude  the rectangle to 3” in height.  (see below)

EXTRUDE

4.  Select the top face  (You must first click the Select Face  button)

5.  Click on the Workplane menu (see below).

6.  Click on New Sketch option.

7. In the dialogue box, Type in the word “Hole” Then press OK

The surface in RED is now activated and ready to draw on.  In the next step you will draw a circle on that surface and “Project” the circle into the part to make a hole.

8.  In the center of the top of the part, draw a circle that is approximately 1 inch diameter (see below)

9.  Click on the “Project Profile” button

10.  In the dialogue box, Check “Subtract material”, “Below Workplane”, “Thru to next face” Click OK

Now that you have a hole through your part try this:

Lets say you only wanted the hole to be a certain depth and NOT go all the way through the part.  If you used the Extrude button  instead of the Project button, you could tell the computer how far you wanted to extrude your circle into the part.  You would have to tell the computer to “Subtract material” “Below the workplane”

**Remember, you have to FIRST create a New Sketch (call it “Hole 2”), THEN draw a circle on the side you selected for the new sketch.

Using what you have learned, select another face and “Extrude a hole INTO the SIDE of the part at a depth of 2” (see below)

Now, roundover one of the edges (see below) If you want to select multiple edged, hold down the Shift key and click of multiple edges.

## TAKE A LOOK AT THE PART YOU CREATED!

Hold down the little roller on the mouse.  While doing this, move your mouse around and see what it does.

Viewing

The following is a quick lesson on the different viewing options (rotating, moving, etc.)

1.  Roll the center wheel on your mouse and observe what happens

2.  Locate the Autoscale  button located on the bottom of the screen and click it.  This puts your part back in the center of the screen

3.  Hold down the center wheel on your mouse and move the mouse around.   This rotates the part

4.  While holding down the shift-key AND holding down the wheel on the mouse, move the mouse around.  This moves the part around.

5.  Hold the shift-key, and the Ctrl-key at the same time while holding down the wheel on the mouse.  This will rotate the part in a circular motion.

Save your work as file name “roundover”

Open your “Test Packet” and answer all questions in Part D

Appendix A

Before you begin, you must change the units from metric to inches.  The following will show you how:

1.  Click on Tools then Options (see below)

2.  Click on the Units tab and change BOTH dropdown menus to inches (see below)

3.  Click OK